GRILLS/OPEN BURNING GUIDELINE

(Disclaimer: This guide is may not be all inclusive, additional regulations may apply) Edited 5/2013

PERMIT SUBMITTAL REQUIREMENTS:

A permit shall be required for BONFIRES, or a CONTROLLED FIRE for recognized silvicultural or range or wildlife management practices.  The permit is obtained by the fire code official:

- Completed application

- Plat of survey(2 copies)

- Show location of proposed fire area on plat.

- Provide proposed burn material(s).

- Provide type and location of fire extinguishing devices.

A permit is also required for any type of exterior fireplaces and or natural gas piped fire containers. The permit is obtained by the building code official:

- Completed application

- Plat of survey (2 copies) with the proposed location of the fire device, with all setback measurements.

- Provide a copy of the manufactures’ installation instructions, if applicable.

- Provide a detail of the gas piping layout; include the proposed depth and materials.

- Provide a detail of the fireplace/chimney etc., if applicable

- Confirm clarify in writing that this proposed structure is NOT in a floodplain, easement, or floodway.

- Provide the Julie Dig Number. (800)-892-0123

GENERAL REQUIREMENTS:

GRILLS (IFC 308)

Charcoal burners and other open flame cooking devices shall not be operated on combustible balconies or within 10 feet of combustible construction.

Propane grills with propane capacity of greater than 1 pound shall not be operated on combustible balconies or within 10’ of combustible construction. See exceptions in the International Fire Code

BONFIRES (IFC 307.3)

A bonfire shall not be conducted within 50 feet of a structure or combustible material unless the fire is contained in a barbecue pit.  Conditions which should cause fire to spread within 50 feet of a structure shall be eliminated prior to ignition.

RECREATIONAL FIRES (IFC 307.3.2)

Recreational fires shall not be conducted within 25 feet of a structure or combustible material.  Conditions which could cause a fire to spread within 25” of a structure shall be eliminated prior to ignition.  See exceptions in the International Fire Code

FIREWOOD

It is recommended that the storage of firewood should be stored in the rear yard, and more than 25’ from any structure and the proposed fire pit. It is also recommended to prevent decay and rodents; store the wood off the ground on an approved container.

APPLICABLE CODES:

- City of Prospect Heights Municipal Ordinance(http://www.prospect-heights.il.us/)

INSPECTION REQUIREMENTS:

- Call 847-398-6070 x 211 to schedule a final inspection